4. Dynamic Routing Configuration using RIP (Routing Information Protocol)

Objective

To configure Dynamic Routing using RIP Version 2 on three Cisco routers and enable automatic route exchange between different networks.

Topology

- 3 Cisco 1941 Router

- 2 Cisco 2960 Switches

- Each switches are connected in 12 PC’s

- Copper Straight-Through Cables

IP Addressing

R1:

- Router G0/0 : 192.168.1.1/24 //gateway of lan1

- Router G0/1 : 10.0.0.1/30

R2:

- Router G0/0 : 10.0.0.2/30

- Router G0/1 : 20.0.0.1/30

R3:

- Router G0/0 : 192.168.2.1/24 //gateway of lan2

- Router G0/1 : 20.0.0.2/30

LAN1

- PC0 : 192.168.1.2/24

- PC1 : 192.168.1.3/24

-PCN: 192.168.1.N/24

LAN2

- PC0 : 192.168.2.2/24

- PC1 : 192.168.2.5/24

-PCN: 192.168.2.N/24

Router Configuration:

Router 1:

enable

configure terminal

interface gigabitethernet0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitethernet0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

end

write

config t

router rip

version 2

network 192.168.1.0

network 10.0.0.0

end

write

Router 2:

enable

configure terminal

interface gigabitethernet0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

interface gigabitethernet0/1

ip address 20.0.0.1 255.255.255.252

no shutdown

exit

end

write

config t

router rip

version 2

network 10.0.0.0

network 20.0.0.0

end

write

Router 3:

enable

configure terminal

interface gigabitethernet0/0

ip address 192.168.2.1   255.255.255.0

no shutdown

exit

interface gigabitethernet0/1

ip address 20.0.0.2  255.255.255.252

no shutdown

exit

end

write

config t

router rip

version 2

network 192.168.2.0

network 20.0.0.0

end

write

Verification Commands

1.show ip interface brief

Verifies that all router interfaces are Up/Up.

2.show ip route

Displays the routing table and confirms that static routes have been successfully added.

3.ping <destination-ip>

Tests connectivity between routers and end devices.

4.show ip protocols

Testing

- Verified that all router interfaces were in up/up state.

- Confirmed RIP Version 2 configuration using show ip protocols.

- Verified dynamically learned routes using show ip route.

- Successfully tested communication between PC1 and PC2 using the ping command.

Key Concepts Learned

- Dynamic Routing Fundamentals

- RIP Version 2 Configuration

- Static vs Dynamic Routing

- Hop Count & Administrative Distance

- Next Hop

- VLSM & CIDR Basics

- RIP Route Verification (show ip route, show ip protocols)

- Routing Table Analysis

- End-to-End Connectivity Testing

Result

Successfully configured Dynamic Routing using RIP Version 2 on three Cisco routers. The routers automatically exchanged routing information, dynamically updated their routing tables, and enabled successful communication between different LANs.

Output: