3. Static Routing Configuration Using Three Routers

Objective

To configure three Cisco routers using static routing and establish successful communication between two different LAN networks through an intermediate router.

Topology

- 3 Cisco 1941 Router

- 2 Cisco 2960 Switches

- Each switches are connected in 9 PC’s

- Copper Straight-Through Cables

IP Addressing

R1:

- Router G0/0 : 192.168.1.1/24 //gateway of lan1

- Router G0/1 : 10.0.0.1/30

R2:

- Router G0/0 : 10.0.0.2/30

- Router G0/1 : 20.0.0.2/30

R3:

- Router G0/0 : 192.168.2.1/24 //gateway of lan2

- Router G0/1 : 20.0.0.1/30

LAN1

- PC0 : 192.168.1.2/24

- PC1 : 192.168.1.3/24

-PCN: 192.168.1.N/24

LAN2

- PC0 : 192.168.2.2/24

- PC1 : 192.168.2.5/24

-PCN: 192.168.2.N/24

Router Configuration:

Router 1:

enable

configure terminal

interface gigabitethernet0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitethernet0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

ip route 192.168.2.0 255.255.255.0 10.0.0.2

end

write

Router 2:

enable

configure terminal

interface gigabitethernet0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

interface gigabitethernet0/1

ip address 20.0.0.2 255.255.255.252

no shutdown

exit

ip route 192.168.1.0 255.255.255.255 10.0.0.1

ip route 192.168.2.0 255.255.255.255 20.0.0.1

end

write

Router 3:

enable

configure terminal

interface gigabitethernet0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface gigabitethernet0/1

ip address 20.0.0.1 255.255.255.252

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 20.0.0.2

end

write

Verification Commands

1.show ip interface brief

Verifies that all router interfaces are Up/Up.

2.show ip route

Displays the routing table and confirms that static routes have been successfully added.

3.ping <destination-ip>

Tests connectivity between routers and end devices.

Testing

Direct Connectivity

Router 1 → Router 2

Router 2 → Router 1

Router 2 → Router 3

Router 3 → Router 2

End-to-End Connectivity

PC0 → PC1

Successful ping confirms that packets are correctly routed through:

Flow:

PC10

↓

Router 1

↓

Router 2

↓

Router 3

↓

PC20

Key Concepts Learned

Static Routing

Multi-hop Routing

Router-to-Router Communication

Point-to-Point Networks (/30)

Next-Hop IP Address

Routing Table Verification

Packet Forwarding

ARP Resolution

End-to-End Connectivity Testing

Network Troubleshooting

Result

Successfully configured three Cisco routers using Static Routing and established communication between two different LAN networks through an intermediate router. Verified connectivity using routing tables and successful end-to-end ping tests.

Output: