1.Basic Router Configuration

Router Basics and Directly Connected Routing

Objective

To configure a Cisco router and enable communication between two different LANs.

Topology

- 1 Cisco 1941 Router

- 2 Cisco 2960 Switches

- Each switch connected 16  PCs

IP Addressing

LAN1

- Router G0/0 : 192.168.1.1/24

- PC0 : 192.168.1.2/24

- PC1 : 192.168.1.3/24

-PCN : 192.168.1.N/24

LAN2

- Router G0/1 : 192.168.2.1/24

- PC2 : 192.168.2.2/24

- PC3 : 192.168.2.5/24

PCN : 192.168.2.N/24

Router Configuration:

->Open the router command line interface

->router> it is normal user command line

1. enable – The router gives the administrative permission

The output: Router#

2.config t / configuration terminal – It set the configuration of the router settings

3.interface gigabitethernet0/0 – we will select the g0/0 for configuration

4.ip address 192.168.1.1 255.255.255.0 – to assign the ip address and also subset for

G0/0

5.no shutdown – default router is shutdown so we say no shutdown

6. exit

7.(LAN -2)select another gigabitethernet1/0 and configure

8.To assign the ip address and subset address

9.no shutdown

10. end – end of the configuration

11.show ip interface brief – configured ip are showed.

Result Successfully configured a Cisco router to enable communication between two LANs using directly connected routing.

Output: